1.	This checklist is used as a supplement to EPM Sections  as applicable.

2.	The Chief Engineer may include / exclude items to meet the requirements of the type of 	machinery.

Date : _____________			Time : _____________		E.O.W. _______________

PREPARATION

Carry out stowaway search and reported to Chief Officer

Maintain F.O. settling & servicing tanks temperature and drain water / sludge

Check jacket cooling water expansion tank

All Double bottom tanks sounded

All bilges transferred as required

Top up Diesel oil tanks and drain off water / sludge

Top up Air bottles and drain off water

AT TEST GEAR (MINIMUM ONE HOUR NOTICE)

At test gear, record all counters

Inform C/E, 2/E & E/E

Test telegraph and steering gear

Test and establish communication on walkie-talkie (C/R to E/R)

T/G idled run and put on load after 15 minutes, if applicable

M/E’s sump sounded before and after starting LO pump

Start M/E auxiliary pumps and check all indicator valves are open

Turn M/E with cylinder oil pumping for 10 mins (with permission of C/E)

Disengage M/E turning gear and check physically

Open main starting air bottle and starting air valve

15 min before standby.  M/E blown thru’ with bridge’s permission

After blown thru’ shut all indicator valves

Start standby generator and share load

M/E auxiliary blowers switched to auto

Warm up deck and bow thruster hydraulic system (2 hrs before use in cold water), if applicable

Start bow thruster fan / pump, if applicable

Start bow thruster as per Master’s requirements, if applicable

At standby, record all counters

Main engine ready for manoeuvring

All auxillary blowers and starting air valves on auto and M/E c/o to wheelhouse control

During manoeuvring, junior watchkeepers take rounds on M/E, A/E and auxillary machinery

Check turbocharger lubrication

Closely monitor M/E performance during change of load

Stop one DG after stopping Bow thruster, if applicable

AFTER FULL AWAY ON PASSAGE

At FAOP, record all counters

For vessel manoeuvring on MDO, open steam heating and c/o to heavy oil and maintain and 	monitor fuel oil viscosity

Check and maintain all M/E parameters during increasing speed

Monitor EGE and M/E operating parameters during loading up

Change sea suction to low suction

Drain water and change over, gradually mix super heat steam, if applicable

EGE soot blown after reaching maximum speed and changed over to auto, if applicable

Main engine speed requested by bridge attained

Start fresh water generator

M/E cylinder lubrication set for FAOP position

Stop one steering pump by bridge

Stop deck hydraulic pumps

Stop deck water

Switch H.O. transfer pump to auto

All M/E parameters checked and in order

___________________________________Chief or 2nd EngineerDuty Engineer__________________Date