1.	This checklist is used as a supplement to EPM Sections as applicable.

2.	The Chief Engineer may include / exclude items to meet the requirements of the type of 	machinery.

Date : ________________	Time : ________________	E.O.W. ____________________

Report to C/R at 0745hrs and take rounds in E/R

Report any abnormality to Senior watchkeeper

Take UMS soundings and record

Carry out M/E turbocharger water washing, if applicable

Drain all pneumatic controllers and reducing valves

Drain H.O. service & settling tanks for water / sludge

Clean fuel oil purifier suction filters

Clean lub oil purifier suction filter as necessary

Check and top up jacket FW expansion tank, DG & EGE seal pots

Top up fresh water generator dosing tank

Check and top up air compressor oil level as required

Clean any oil patch found on floor plate

Drain control air drier, air bottles and air lines

Take noon log and record

Any topping up of lub oil is recorded

Collect data logger printout and submit to C/E with E/R log book

Lower bilge and sludge tank level

Empty rubbish into larger bin near incinerator

Clean bilge line filters as required

Keep sufficient rags and waste in workshop

Clean and arrange workship and tools

Take UMS sounding and prepare for UMS

Assist senior watchkeeper in testing UMS alarm (at least two)

Assist senior watchkeeper in night UMS checks and fire rounds at 2200hrs